# Methods

## 1. Data Sources

### FloodNet Water-Depth Sensors

We used the FloodNet network of distributed water-level sensors deployed across New York City. For each device, we obtained:

- A unique deployment_id, sensor name, latitude, and longitude.

- High-frequency water-depth observations (mm), retrieved through the FloodNet API.

These raw measurements were temporally aligned and aggregated to hourly depth values for analysis.

### Radar-Based Precipitation

Precipitation forcing was obtained from the NOAA Multi-Radar Multi-Sensor (MRMS) system stored in Zarr format. MRMS provides hourly precipitation at ~1 km resolution. For each FloodNet sensor, the nearest MRMS grid cell was identified using a great-circle distance search, and hourly precipitation time series were extracted for the full study period.

### NYC 311 Service Requests

The NYC 311 dataset was used to characterize community-reported flooding and drainage issues. Relevant fields included:

- created_date

- complaint_type and descriptor

- latitude and longitude

We made use of two categories:

- Street Flooding complaints, identified by descriptors containing “street” and “flood.”

- Catch Basin complaints, grouped into a broader complaint type (“Catch_Basin_Issues”).

### Topographic and Built-Environment Layers

Three geospatial datasets were used to characterize the physical environment around sensors:

- Digital Elevation Model (DEM) raster for NYC.

- Slope raster, derived from the DEM.

- NYC Building Footprints, a polygon layer used to compute distance-to-building metrics.

### NRI Census Tract Layer

We used the National Risk Index (NRI) census-tract polygons as the unit for downstream aggregation. These polygons receive sensor-derived flooding sensitivity scores and are used for spatial comparison across the city.

### Synthetic Sensors

A grid of 500 regularly spaced “virtual sensors” was generated to fill spatial gaps between real sensors. These points inherit hydrologic behavior from nearby real sensors using topographic similarity.

## 2. Data Processing Workflow (Python / Jupyter)

### 2.1. Preprocessing Water Depth

All depth observations were converted to UTC timestamps and merged into df_depth_hourly, an hourly time series for every FloodNet sensor. This ensured temporal alignment with the hourly MRMS precipitation data.

### 2.2. Preprocessing Precipitation

For each deployment location, the nearest MRMS grid cell was selected. The extracted hourly precipitation was appended to the depth time series. Merging depth and precipitation by deployment_id and time_utc produced a unified hourly table (merged_all) serving as the foundation for daily hydrology and complaint analyses.

## 3. Linking 311 Street-Flooding Complaints to Sensors

### 3.1. Filtering and Temporal Alignment

Street-flooding complaints were extracted using descriptor matching (“street” AND “flood”).
Each 311 record was:

- Converted to datetime,

- Filtered to valid (lat, lon),

- Rounded down to the nearest hour, matching the hourly sensor resolution.

### 3.2. Spatial Association (within 0.5 km)

Sensor coordinates were converted to radians and loaded into a BallTree using haversine distance.
Each 311 street-flooding record was assigned to all sensors within 0.5 km radius.
This produced a table of hourly complaint counts per sensor (sf_count_neighboring).

### 3.3. Daily Aggregation on Wet Days

Hourly precipitation and complaint counts were aggregated to daily values:

- daily_precip_mm = sum of hourly precipitation.

- daily_sf = total street-flooding complaints within 0.5 km.

Only wet days (daily_precip_mm > 0) were retained.
For each sensor, the mean daily street-flooding complaints on wet days was computed (sf_avg_wet_days).

## 4. Catch Basin Complaints (Daily Wet-Day Metric)

Catch Basin complaints were grouped under a unified descriptor class (Catch_Basin_Issues).
Daily complaint counts were merged with daily precipitation totals.
Restricting to wet days, the mean daily CB complaints per wet day was calculated (cb_avg_wet_days).

## 5. Rain-Sensitivity Analysis (Negative Binomial GLM)

To assess rain response for each sensor, a Negative Binomial generalized linear model (NBGLM) was fitted at the daily scale using:

- Depth (counts) as the dependent variable,

- Daily precipitation plus 311 complaint terms as predictors.

For each sensor, a rain-slope coefficient (coef_precip) and p-value were extracted.
A sensor was deemed rain-sensitive if:

- p_value < 0.05, and

- coef_precip > 0.

Approximately 55 sensors met this criterion.

## 6. Constructing the Master "Existing Sensors with Rain" Dataset

The following fields were merged per deployment_id:

- Sensor coordinates and name,

- coef_precip, p_value, n_obs,

- Sensitive / not sensitive indicator,

- sf_avg_wet_days,

- cb_avg_wet_days,

- Indicator for having any rainy days.

This table was later enriched with topographic data (elevation, slope, distance to buildings).

## 7. Topographic Processing (ArcGIS Pro)

### 7.1. Extracting Elevation and Slope

Using Spatial Analyst → Extract Multi Values to Points, DEM values and slope raster values were assigned to each sensor.

### 7.2. Distance to Nearest Building

Using Proximity → Near:

- Input: sensor points

- Near features: building footprint polygons
ArcGIS added:

- NEAR_FID

- NEAR_DIST (distance to nearest building polygon)

These fields were exported back to Jupyter for normalization.

### 7.3. Topographic Normalization

In Python:

- Convert <Null> to NaN, cast to numeric.

- Remove rows with negative values.

- Min–max normalize Elevation, Slope, and Near-Distance to [0,1].

The resulting fields were elev_norm, slope_norm, near_norm.

## 8. Scoring Framework

### 8.1. Precipitation Sensitivity Score (score_precip)

- If p_value ≥ 0.05, or coef_precip ≤ 0 → score_precip = 0.

- If coef_precip = 0 → score_precip = 3.

- If p_value < 0.05 and coef_precip > 0 → normalize positive coefficients to the 4–5 range.

### 8.2. Street Flooding Score (score_sf)

- If sf_avg_wet_days = 0 → 0.

- Otherwise, normalize positive values to the 3–4 range.

### 8.3. Catch Basin Complaint Score (score_cb)

- Same procedure as score_sf.

### 8.4. Total Score

score_total = score_precip + score_sf + score_cb.

This produced an interpretable hazard-sensitivity profile for each sensor.

## 9. Synthetic Sensors and Propagation of Scores Citywide

### 9.1. Synthetic Sensor Creation (ArcGIS Pro)

A 500-point grid was generated over NYC using:

- Create Fishnet → Feature To Point → Clip to Study Area.

Elevation, slope, and near-building distances were extracted using the same workflows as for real sensors.

### 9.2. Topo-Space Nearest Neighbor Assignment

In Python:

- Normalize topo variables for synthetic sensors.

- Compute nearest existing sensor in (elev_norm, slope_norm, near_norm) space.

- Assign the existing sensor's score_precip, score_sf, score_cb, score_total to the synthetic sensor.

This produced a spatially continuous surrogate flood-responsiveness surface.

## 10. Score Propagation to Census Tracts (ArcGIS Pro)

### 10.1. Merge Real and Synthetic Sensors

Both sensor layers were appended into a single point dataset containing four score fields.

### 10.2. Tract-Level Assignment

Using Spatial Join:

- Target: NRI census tracts

- Join features: combined sensor points

- Mode: CLOSEST

Each tract was assigned:

- score_precip

- score_sf

- score_cb

- score_total

from the nearest sensor (ensuring full spatial coverage even if no sensors were inside the polygon boundaries).

# Summary

This integrated multi-source workflow combines:

- High-resolution hydrologic sensing,

- Radar precipitation,

- Community-reported flooding,

- Built-environment and topographic data,

- Statistical modeling (NBGLM),

- Spatial interpolation using synthetic sensors,

- Census-tract aggregation using GIS.

The result is a robust spatially continuous flood-sensitivity index reflecting hydrologic behavior, environmental context, and community impact pathways.

1. Data sources

- FloodNet sensor network

  - Point sensors across NYC measuring water depth (mm) at high frequency.

  - API used to retrieve:

    - Deployment metadata: deployment_id, sensor name, latitude, longitude.

    - Depth time series for each sensor.

- Radar-based precipitation

  - Hourly MRMS / similar radar precipitation product stored as Zarr.

  - Variables used: hourly precipitation depth (mm) on a regular lat–lon grid.

- NYC 311 service requests

  - Full 311 dataset for the study period.

  - Fields used: created_date, complaint_type, descriptor, latitude, longitude, plus some basic categorical metadata.

- Topographic and built-environment data

  - Elevation raster: DEM covering NYC.

  - Slope raster: derived from the DEM.

  - Building footprints: polygon layer of all NYC buildings.

- NRI census-tract polygons

  - Polygon layer in which each row is a census tract with NRI attributes.

- Synthetic (“virtual”) sensors

  - 500 regularly spaced points covering NYC (created in ArcGIS Pro).

2. Depth & precipitation processing (Python / Jupyter)

2.1. Sensor metadata

- Query FloodNet API for all deployments.

- Build df_deployments with:

  - deployment_id, name, latitude, longitude, and other fields.

- Define the subset of deployments actually used (df_used_coordinates).

2.2. Water depth time series

- For each deployment_id in df_used_coordinates:

  - Download depth data over the analysis period.

  - Convert timestamps to UTC and sort by time.

  - Aggregate to hourly depth:

    - Column: depth_proc_mm (e.g., mean or max depth in that hour).

- Stack all sensors into a single dataframe df_depth_hourly:

  - Columns: deployment_id, time_utc, depth_proc_mm.

2.3. Radar precipitation time series

- Open MRMS Zarr datasets for the relevant years.

- Determine the lat–lon bounds of all sensors.

- For each sensor:

  - Identify nearest MRMS grid cell indices (iy, ix) using great-circle distance.

  - Extract hourly precipitation for that grid cell for the sensor’s time range.

- Build radar_df_all with:

  - deployment_id, time_utc, radar_precip_mm.

2.4. Merge depth and precipitation

- Merge df_depth_hourly and radar_df_all on deployment_id and time_utc:

- merged_all = df_depth_hourly.merge(

- radar_df_all,

- on=["deployment_id", "time_utc"],

- how="inner"

- )

- Remove rows with missing depth or precipitation.

- This merged_all dataframe is the hourly master table, used later for:

  - Daily aggregations.

  - Linking to 311 complaints.

  - Lagged precipitation analyses.

3. Daily aggregation and initial NBGLM sensitivity

3.1. Daily sensor hydrology

- From merged_all:

- df = merged_all.copy()

- df["date"] = df["time_utc"].dt.date

- daily = (

- df.groupby(["deployment_id", "date"])

- .agg(

- daily_depth_mm=("depth_proc_mm", "sum"),

- daily_precip_mm=("radar_precip_mm", "sum")

- )

- .reset_index()

- )

- Each row of daily is a sensor–day pair with total water depth and total rainfall.

3.2. 311 → sensor linking for all descriptors (for NBGLM)

(This was the earlier complaint work that fed into the NBGLM)

- Read 311 data as df_311, filter to the study period.

- Convert to GeoDataFrames (GeoPandas):

  - Sensors: gdf_sensors from df_used_coordinates.

  - 311 points: gdf_311 from df_311[["latitude","longitude"]].

- Project to a local projected CRS, e.g. EPSG:2263 (NAD 1983 StatePlane NY Long Island).

  - gdf_sensors_2263, gdf_311_2263.

- Spatial join within 1 km:

  - Use a buffer or distance join so that each 311 complaint is associated with any sensor within ~1 km (you used a feet equivalent constant ONE_KM_FEET).

- Aggregate by hour and descriptor to get hourly/daily complaint counts per sensor.

- Pivot to wide format:

  - Columns like compl_Flooding_on_Street, compl_Catch_Basin_Issues, etc.

3.3. Negative Binomial GLM (daily)

- Merge daily with the wide complaint table to form df_reg.

- Remove rows with missing daily_depth_mm or daily_precip_mm.

- Create an integer “count” version of depth:

- df_reg_nb = df_reg.copy()

- df_reg_nb["depth_count"] = (

- df_reg_nb["daily_depth_mm"]

- .clip(lower=0)

- .round()

- .astype("int64")

- )

- Clean column names for Patsy:

- df_reg_nb = df_reg_nb.rename(

- columns=lambda c: re.sub(r"[^0-9a-zA-Z_]+", "_", c)

- )

- Build formula:

- complaint_cols = [c for c in df_reg_nb.columns if c.startswith("compl_")]

- predictors = ["daily_precip_mm"] + complaint_cols

- formula = "depth_count ~ " + " + ".join(predictors)

- Fit NB GLM using statsmodels:

- model_nb = smf.glm(

- formula=formula,

- data=df_reg_nb,

- family=sm.families.NegativeBinomial()

- )

- result_nb = model_nb.fit()

- Extract per-sensor coefficients (you looped over sensors or refit by deployment_id to get sensor-specific coef_precip, p_value, and n_obs), building df_nb_results.

3.4. Defining “rain-sensitive” sensors (daily scale)

- A sensor is labelled rain-sensitive if:

  - p_value < 0.05

  - coef_precip > 0

- This yields a subset of ~55–60 “sensitive” sensors.

- The same df_nb_results also used later for scoring.

4. Street-flooding complaints (hourly and daily)

4.1. Select street-flooding records

df_311_street = df_311[

df_311["descriptor"].str.contains("street", case=False, na=False)

& df_311["descriptor"].str.contains("flood", case=False, na=False)

].copy()

- Convert and clean:

  - created_date → datetime.

  - Drop rows with missing created_date, latitude, or longitude.

  - Floor to the hour: time_utc_hour = created_date.dt.floor("H").

4.2. Spatial linking to sensors via BallTree (0.5 km radius)

- Build a unique sensor coordinate table from df_used_coordinates.

- Convert to radians and build a haversine BallTree:

- sensor_coords_rad = np.radians(df_sensors[["latitude","longitude"]].values)

- tree = BallTree(sensor_coords_rad, metric="haversine")

- For each street-flooding complaint:

  - Convert its lat/lon to radians.

  - Query sensors within 0.5 km:

  - radius_km = 0.5

  - radius_rad = radius_km / earth_radius_km

  - ind = tree.query_radius([[lat_rad, lon_rad]], r=radius_rad)[0]

  - For each nearby sensor index, record a (deployment_id, time_utc_hour) pair.

- Build sf_counts_hourly:

- df_pairs = pd.DataFrame(pairs, columns=["deployment_id", "time_utc"])

- sf_counts_hourly = (

- df_pairs.groupby(["deployment_id", "time_utc"])

- .size()

- .reset_index(name="sf_count_neighboring")

- )

4.3. Merge SF counts into hourly sensor data

- Left-join onto merged_all:

- df_hourly_with_311 = merged_all.merge(

- sf_counts_hourly,

- on=["deployment_id", "time_utc"],

- how="left"

- )

- df_hourly_with_311["sf_count_neighboring"] = (

- df_hourly_with_311["sf_count_neighboring"]

- .fillna(0)

- .astype(int)

- )

4.4. Daily SF metric on wet days

- Aggregate to daily:

- df_hour = df_hourly_with_311.copy()

- df_hour["date"] = df_hour["time_utc"].dt.date

- daily_sf = (

- df_hour.groupby(["deployment_id", "date"])

- .agg(

- daily_precip_mm=("radar_precip_mm", "sum"),

- daily_sf=("sf_count_neighboring", "sum")

- )

- .reset_index()

- )

- Keep only wet days (daily_precip_mm > 0):

- daily_wet = daily_sf[daily_sf["daily_precip_mm"] > 0]

- Compute mean daily SF per sensor on wet days:

- sf_avg_wet = (

- daily_wet.groupby("deployment_id")["daily_sf"]

- .mean()

- .reset_index(name="sf_avg_wet_days")

- )

5. Catch-basin complaints (daily wet-day metric)

- Earlier in the workflow, you grouped 311 descriptors into broader categories and built a complaints_daily table with fields:

  - deployment_id, date, descriptor_group, complaint_count.

- One of those groups was "Catch_Basin_Issues" (aggregating all catch-basin-relevant descriptors).

- Steps:

- cb = complaints_daily[

- complaints_daily["descriptor_group"] == "Catch_Basin_Issues"

- ].copy()

- cb["complaint_count"] = cb["complaint_count"].fillna(0)

- Join with daily_precip to identify wet days:

- daily_precip = (

- df_p.groupby(["deployment_id", "date"])["radar_precip_mm"]

- .sum()

- .reset_index(name="daily_precip_mm")

- )

- cb_join = daily_precip.merge(

- cb[["deployment_id","date","complaint_count"]],

- on=["deployment_id","date"],

- how="left"

- )

- cb_join["complaint_count"] = cb_join["complaint_count"].fillna(0)

- cb_wet = cb_join[cb_join["daily_precip_mm"] > 0]

- Compute mean daily catch-basin complaints on wet days:

- cb_avg_wet = (

- cb_wet.groupby("deployment_id")["complaint_count"]

- .mean()

- .reset_index(name="cb_avg_wet_days")

- )

6. Building the per-sensor “Existing_Sensors_with_Rain” table

- Start from df_used_coordinates (deployment_id, name, latitude, longitude).

- Merge in:

  - NBGLM results: coef_precip, p_value, n_obs.

  - Boolean is_sensitive = (coef_precip>0 & p_value<0.05).

  - has_rain_day flag from daily precip (whether sensor ever had daily_precip_mm > 0).

  - sf_avg_wet_days from step 4.

  - cb_avg_wet_days from step 5.

  - Later: topographic variables from ArcGIS (sfcbElevationRAster, sfcbSlopeRaster, NEAR_DIST).

- This is essentially the Existing_Sensors_with_Rain.xlsx file.

7. Scoring scheme (precip sensitivity, SF, CB)

Using Existing_Sensors_with_Rain:

7.1. Precipitation coefficient score (score_precip)

For each sensor:

- If p_value is null, or coef_precip is null → score_precip = 0.

- Else if p_value >= 0.05 → score_precip = 0.

- Else (p_value < 0.05):

  - If coef_precip < 0 → 0.

  - If coef_precip == 0 → 3.

  - If coef_precip > 0:

    - Let min_pos and max_pos be the min/max of positive coef_precip over all sensors.

    - Compute normalized value:

z=coef_precip-min⁡_posmax⁡_pos-min⁡_pos

    - Map to the range 4–5:

score_precip=4+z×1

7.2. Street-flooding score (score_sf)

- If sf_avg_wet_days is 0, null, or blank → score_sf = 0.

- For sensors with sf_avg_wet_days > 0:

  - Find min_sf, max_sf over positive values.

  - Normalize:

z=sf_avg_wet_days-min⁡_sfmax⁡_sf-min⁡_sf

  - Map linearly to 3–4:

score_sf=3+z×1

7.3. Catch-basin score (score_cb)

- Identical procedure, using cb_avg_wet_days instead of sf_avg_wet_days.

7.4. Total score

- score_total = score_precip + score_sf + score_cb.

These four score columns were added to the existing-sensor table.

8. Topographic processing (ArcGIS Pro) and normalization

8.1. For existing sensors

- Load existing sensor CSV (with scores) into ArcGIS Pro and make it a point layer via Display XY.

- Set/confirm CRS for the map to a projected CRS consistent with your elevation raster and building footprints (e.g., NY StatePlane feet).

- Elevation:

  - Use Spatial Analyst → Extraction → Extract Multi Values to Points (or Sample) with the DEM as input.

  - New field added (e.g. sfcbElevationRAster).

- Slope:

  - If not already available, create a slope raster from the DEM:

    - Spatial Analyst → Surface → Slope (in degrees).

  - Use Extract Multi Values to Points to add sfcbSlopeRaster.

- Distance to nearest building:

  - Use Analysis → Proximity → Near:

    - Input Features: sensor points.

    - Near Features: building footprints.

  - This adds fields NEAR_FID, NEAR_DIST (in CRS units).

- Export the attribute table as Existing_Sensors_with_Rain_topo (Excel/CSV).

8.2. Topo normalization for existing sensors (Python)

- Read the exported file in Jupyter.

- Topo columns: sfcbElevationRAster, sfcbSlopeRaster, NEAR_DIST.

- Clean:

  - Convert "<Null>" strings to NaN.

  - Cast to numeric.

  - Drop rows with any negative topo values.

- For each topo column:

  - Compute min and max.

  - If max == min → assign 0.5 to all (degenerate case).

  - Else:

norm=x-min⁡max⁡-min⁡

- Add columns like elev_norm, slope_norm, near_norm (all in [0,1]).

- Save as Existing_Sensors_with_Rain_topo_norm.xlsx.

8.3. Synthetic sensors: creation & topo extraction

- Create 500 evenly spaced points in NYC:

  - In ArcGIS Pro:

    - Use Create Fishnet or Generate Tessellation to create a regular grid covering NYC.

    - Use Feature To Point to convert grid cells to centroids.

    - Clip to NYC boundary if necessary.

- Sample DEM, slope, buildings:

  - Repeat the same Extract Multi Values to Points and Near steps as for existing sensors, generating fields:

    - ElevationR, SlopeRaste, NEAR_DIST.

- Export as new_sensors_500_topo_factors.csv.

8.4. Topo normalization for synthetic sensors (Python)

- Read new_sensors_500_topo_factors.csv.

- Convert <Null> to NaN, cast to numeric, drop negative topo values.

- For columns ElevationR, SlopeRaste, NEAR_DIST:

  - Min–max scale each to [0,1] (same pattern).

- Save as new_sensors_500_topo_normalized.csv.

9. Assigning scores to synthetic sensors (nearest neighbour in topo space)

- Load:

  - Existing sensors with scores and normalized topo.

  - New sensors with normalized topo.

- Define feature matrices:

- exist_X = df_exist[["elev_norm", "slope_norm", "near_norm"]].values

- new_X   = df_new[["elev_norm", "slope_norm", "near_norm"]].values

- Use a nearest-neighbor search (e.g., BallTree or cdist) in this 3-D normalized topo space:

  - For each synthetic sensor, find the closest existing sensor.

  - Copy over score_precip, score_sf, score_cb, score_total, and the source deployment_id.

- Save as e.g. new_sensors_with_scores.csv.

At this point you have two point layers with identical scoring scheme:

- Existing FloodNet sensors with scores & topo.

- Synthetic sensors (virtual points) with scores derived from nearest existing sensors in topo space.

10. Propagating scores to NRI census tracts (ArcGIS Pro)

You combined existing and synthetic sensors, then assigned tract-level scores.

- Add both point layers (existing+synthetic) into the same ArcGIS Pro map.

- Ensure both layers use the same projected CRS as the NRI tract polygons.

- Optionally, merge the two sensor layers:

  - Use Data Management → General → Append or Merge.

  - Result: one point feature class with all sensors and fields:

    - score_precip, score_sf, score_cb, score_total.

- Spatial Join (closest point → polygon):

  - Analysis → Overlay → Spatial Join.

  - Target Features: NRI census-tract polygon layer.

  - Join Features: merged sensor point layer.

  - Join Operation: JOIN_ONE_TO_ONE.

  - Match Option: CLOSEST (so every tract, even if empty, receives the scores from its nearest sensor).

  - For the score fields:

    - Use MAX or FIRST as the merge rule—because with CLOSEST there will effectively be only one point per tract in the join result.

- Output: a new NRI tract layer with four new fields:

  - score_precip

  - score_sf

  - score_cb

  - score_total
each representing the scores from the closest sensor (existing or synthetic).

11. What you now have

For each census tract:

- A precipitation sensitivity score informed by:

  - FloodNet sensor response to rainfall (NBGLM on daily data).

- A street-flooding complaint score based on:

  - Average daily street-flooding complaints on wet days within 0.5 km of nearby sensors.

- A catch-basin complaint score based on:

  - Average daily catch-basin complaints on wet days near sensors.

- An overall combined score (score_total = score_precip + score_sf + score_cb),
propagated from the nearest sensor whose physical context is approximated for unsensed locations with the synthetic sensor grid.

And for each sensor (existing or synthetic), you have:

- Location, topo features (elevation, slope, distance to buildings).

- Rain sensitivity (coef_precip, p_value, is_sensitive).

- SF and CB wet-day complaint intensities.

- Normalized scores and topo normals useful for further ML (e.g., GNNs).